EdgeSense

Cloud-Native Edge AI Anomaly Detection Platform

MSc Placement Project Proposal

MSc Software Design (Cloud-Native Computing) | Technological University of the Shannon, Athlone

May 2026

# 1. Project Overview

EdgeSense is a cloud-native IoT monitoring system that combines edge-based AI inference with AWS cloud services to detect anomalies in real-time environmental sensor data. A lightweight AI model runs locally on a Raspberry Pi, analysing temperature and humidity readings captured by an Arduino Uno. When an anomaly is detected, the result is streamed to AWS where a suite of microservices handles storage, alerting, and visualisation.

The system is designed around modern cloud-native principles: containerised microservices, automated CI/CD pipelines, and scalable cloud infrastructure — directly aligned with the MSc in Software Design (Cloud-Native Computing).

# 2. Project Objectives

- Design and implement a cloud-native microservices architecture on AWS

- Deploy a lightweight AI anomaly detection model on a Raspberry Pi edge device

- Integrate an Arduino Uno with a DHT22 sensor for real-time temperature and humidity capture

- Stream anomaly events from the edge to AWS IoT Core using MQTT

- Build a real-time dashboard to visualise sensor readings and anomaly events

- Implement a fully automated CI/CD pipeline using GitHub Actions and Docker

- Support over-the-air (OTA) model updates pushed from cloud to edge device

# 3. System Architecture

## 3.1 Hardware Layer

The hardware stack consists of two devices working in combination:

| Device | Role |
| --- | --- |
| Arduino Uno | Reads temperature and humidity from DHT22 sensor; sends data over USB serial |
| Raspberry Pi | Receives serial data; runs AI anomaly detection model; communicates with AWS |

## 3.2 Edge Layer

The Raspberry Pi acts as the edge computing node. It receives raw sensor readings from the Arduino over USB serial and passes them through an Isolation Forest machine learning model. This model has been trained on normal operating ranges and flags deviations as anomalies. Processing at the edge ensures low-latency detection and continued operation even when internet connectivity is unavailable.

## 3.3 Cloud Layer (AWS)

Anomaly events are published via MQTT to AWS IoT Core. A pipeline of containerised Spring Boot microservices then processes, stores, and acts on each event:

| Microservice | Responsibility |
| --- | --- |
| Ingestion Service | Receives incoming events from AWS IoT Core |
| Anomaly Processing Service | Validates events and enriches with metadata |
| Notification Service | Triggers email or SMS alerts via AWS SNS |
| Storage Service | Persists sensor readings and anomaly events to database |
| Model Management Service | Manages AI model versions; pushes updates to edge device |
| API Gateway | Exposes unified REST API to the dashboard frontend |

# 4. Technology Stack

| Category | Technology |
| --- | --- |
| Edge AI | Python, scikit-learn (Isolation Forest), ONNX |
| Edge Communication | MQTT, pyserial |
| Microservices | Java, Spring Boot, Docker, AWS ECS |
| Cloud Platform | AWS IoT Core, Kinesis, Lambda, SNS, DynamoDB, ECR |
| CI/CD | GitHub Actions, Docker Hub |
| Dashboard | React, Grafana |
| Version Control | Git, GitHub |

# 5. AI / Machine Learning Component

The anomaly detection model is based on the Isolation Forest algorithm — an unsupervised machine learning method well suited to detecting outliers in continuous sensor data. The model is trained on a baseline dataset of normal temperature and humidity readings collected during an initial calibration period.

Key properties of this approach:

- Runs efficiently on constrained hardware (Raspberry Pi) without GPU

- No labelled training data required — learns from normal operating patterns

- Produces an anomaly score for each reading, allowing configurable sensitivity thresholds

- Model can be retrained and redeployed OTA from the cloud as new data is collected

This approach builds on prior research comparing DepthAnything V2 and MiDaS models for edge inference, applying similar edge deployment principles to a real-time monitoring context.

# 6. Development Phases

| Phase | Milestone | Deliverable |
| --- | --- | --- |
| 1 | Environment Setup | GitHub repo, AWS account, Docker, local dev environment |
| 2 | Hardware Integration | Arduino DHT22 readings streamed to Raspberry Pi via serial |
| 3 | Edge AI Model | Isolation Forest model trained and running on Raspberry Pi |
| 4 | Cloud Microservices | Spring Boot services deployed on AWS ECS via Docker |
| 5 | MQTT Integration | Edge-to-cloud pipeline live via AWS IoT Core |
| 6 | Dashboard | Real-time anomaly dashboard built and connected to API |
| 7 | CI/CD Pipeline | Automated build and deploy pipeline via GitHub Actions |
| 8 | OTA Model Updates | Model Management Service pushing updates to edge device |

# 7. Summary

EdgeSense is a well-scoped, technically ambitious placement project that demonstrates practical application of cloud-native computing, AI at the edge, and microservices architecture. It is realistic within a placement timeframe, incrementally deliverable across defined phases, and produces tangible outputs at each stage. The hardware stack (Raspberry Pi + Arduino Uno + DHT22 sensor) is low cost and readily available, minimising infrastructure risk.

This project provides a strong foundation for the MSc dissertation and produces a portfolio piece directly relevant to software engineering roles in cloud, IoT, and AI domains.